METRIX MTX329x Multimeter

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for METRIX MTX329x Multimeter.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

METRIX MTX329x Multimeter

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                    Function Name:

Initialize                        mtx329x_init

Configure

Voltage                        mtx329x_voltage

Current                        mtx329x_current

Frequency                      mtx329x_frequency

Resistance                     mtx329x_resistance

Capacitance                    mtx329x_capacitance

Temperature                    mtx329x_temperature

Mode

Filter                         mtx329x_filter

Impedance                      mtx329x_impedance

dBm Ref                        mtx329x_dBmRef

Watt Ref                       mtx329x_WattRef

Measurement                       mtx329x_measure

Error                             mtx329x_error

Close                             mtx329x_close

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

mtx329x_capacitance

int mtx329x_capacitance (int portCom, int range);

Purpose

Switches the instrument to Capacitance measurement.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

range

Variable Type       int

Specifies the range of the measurement.

Variable Type:  integer

Variable Range:  0 -  Auto

1 -  1000 pF

2 -  10 nF

3 -  100 nF

4 -  1000 nF

5 -  10 F

6 -  100 F

7 -  1000 F

8 -  10 mF

Default Value:   0 - Auto Range

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_close

int mtx329x_close (int portCom);

Purpose

Closes the instrument.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_current

int mtx329x_current (int portCom, int function, int range);

Purpose

Switches the instrument to current measurement.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 - Auto

1 - 1000 A

2 - 10 mA

3 - 100 mA

4 - 1000 mA

5 - 10 A

6 - 20 A

Default Value:   0 - Auto

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_dBmRef

int mtx329x_dBmRef (int portCom, int dBmReference);

Purpose

Changes the Filter of the instrument.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

dBmReference

Variable Type       int

Specifies the value of reference in dBm Measurement

Variable Type:  integer

Variable Range:  1 to 10000 Ohm

Default Value:   600 Ohm

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_error

int mtx329x_error (int portCom, char errorMessage[]);

Purpose

Returns the code error

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

errorMessage

Variable Type       char []

Specifies the buffer to write the error message.

Variable Type:  char *

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_filter

int mtx329x_filter (int portCom, int filter);

Purpose

Changes the Filter of the instrument.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

filter

Variable Type       int

Specifies the filter of the instrument

Variable Type:  integer

Variable Range:  1 - On

0 - Off

Default Value:   0 - Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_frequency

int mtx329x_frequency (int portCom, int mode, int range);

Purpose

Sets the instrument to frequency measurement.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

mode

Variable Type       int

Specifies the mode of measurement.

Variable Type: integer

Valid Range:   0 -  <200 kHz

1 -  >200 kHz

Default Value: 0 -  <200kHz

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  1 - 100 mV

2 - 1000 mV

3 - 10 V

4 - 100 V

5 - 1000 V

Default Value:   1 - 100 mV

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_impedance

int mtx329x_impedance (int portCom, int impedance);

Purpose

Changes the Filter of the instrument.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

impedance

Variable Type       int

Specifies the impedance of the instrument

Variable Type:  integer

Variable Range:  1 - 1 GOhm

0 - 10/20 MOhm

Default Value:   0 - 10/20 MOhm

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_init

int mtx329x_init (int portCom, int IDQuery, int resetDevice);

Purpose

Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument

- optionally performs an

identification query on the

instrument

- optionally sets the instrument

to a default configuration

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

IDQuery

Variable Type       int

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

resetDevice

Variable Type       int

Control Name:  Reset

Description:   Specifies if the instrument is to

be reset during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 = No Reset

1 = Reset device

Default Value: 1 = Reset

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_measure

int mtx329x_measure (int portCom, double *measure);

Purpose

Returns the measurements of the instrument.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

measure

Variable Type       double (passed by reference)

Displays the results of the measurement in

Scientific Format.

Variable Type:  double

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_resistance

int mtx329x_resistance (int portCom, int measure, int range);

Purpose

Sets the instrument to Resistance measurement.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

measure

Variable Type       int

Specifies the measurement

Variable Type:  integer

Variable Range:  0 -  Resistance

1 -  Continuity testing

2 -  100 Ohm

3 -  Standard diode testing

4 -  Special diode testing

Default Value:   0 -  Resistance

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 -  Auto

1 -  1000  Ohm

2 -  10 kOhm

3 -  100 kOhm

4 -  1000 kOhm

5 -  10 MOhm

6 -  100 MOhm

Default Value:   0 - Auto

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_temperature

int mtx329x_temperature (int portCom, int unit, int sensor);

Purpose

Sets the instrument to Temperature measurement.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

unit

Variable Type       int

Specifies the unit of temperature.

Variable Type:  integer

Variable Range:  0 - C

1 - F

2 - K

Default Value:   0 - C

sensor

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 -  Pt100

1 -  Pt1000

2 -  TC J

3 -  TC K

Default Value:   0 - Pt100

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_voltage

int mtx329x_voltage (int portCom, int function, int range);

Purpose

Switches the instrument to voltage measurement.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

3 -  VlowZ

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 - Auto

1 - 100 mV

2 - 1000 mV

3 - 10 V

4 - 100 V

5 - 1000 V

Default Value:   0 - Auto

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mtx329x_WattRef

int mtx329x_WattRef (int portCom, int wattReference);

Purpose

Changes the Filter of the instrument.

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

wattReference

Variable Type       int

Specifies the value of reference in Watt Measurement

Variable Type:  integer

Variable Range:  1 to 10000 Ohm

Default Value:   50 Ohm

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4